# 1. Project overview

This project will design, configure, and simulate a reliable, scalable, and secure network for Urban Mart, comprising a main site and a connected branch office, using Cisco Packet Tracer. The simulation will include PCs, printers, switches, routers, and cloud connectivity to represent WAN routing between sites. Key goals are smooth inter-departmental communication, secure Internet access for guests, and policy enforcement to protect internal resources.

# 2. Project objectives

- Design a scalable LAN architecture for both main and branch sites.

- Simulate end-to-end connectivity between PCs, printers, servers, and Internet cloud.

- Configure core services (DHCP, DNS, HTTP) on simulated servers.

- Implement VLANs to logically separate departments and guest access.

- Enforce security through ACLs and port security.

- Establish inter-site routing via the cloud for failover and redundancy.

- Document all configurations, addressing, and topology for future deployment.

# 3. Project Scope

## In-scope:

- Logical & physical network design in Cisco Packet Tracer.

- Configuration of routers (Cisco 2911), switches (2960 & 3650), PCs, printers, and servers.

- VLAN segmentation and inter-VLAN routing.

- WAN emulation using the cloud object and static/dynamic routing.

- Basic security (ACLs, port security, DHCP snooping).

- Functional testing and troubleshooting in simulation.

## Out-of-scope:

- Physical cabling and hardware installation.

- Real-world ISP provisioning or wireless deployment.

# 4. TOOLS AND TECHNOLOGIES

- Simulation: Cisco Packet Tracer

- Routers: Cisco 2911 (Main and Branch)

- Switches: Cisco Catalyst 2960 (Access), Cisco Catalyst 3650 (Distribution)

- End Devices: PCs (pc_0–pc_8), Printers (printer_0–printer_7), Servers (Web, DNS, DHCP, File)

- Cloud: Packet Tracer Cloud object for WAN

- Protocols: DHCP, DNS, HTTP, VLANs, OSPF/RIP, ACLs, Port Security

# 5. NETWORK REQUIREMENTS

Hardware (Simulation Only)

- 2 × Cisco 2911 Routers (Main-site & Branch-site)

- 2 × Cisco Catalyst 3650 Switches (Distribution at each site)

- 2 × Cisco Catalyst 2960 Switches (Access at each site)

- 9 × PCs (pc_0 through pc_8)

- 8 × Printers (printer_0 through printer_7)

- 4 × Servers (Web, DNS, DHCP, File)

- 1 × Packet Tracer Cloud object

# LOGICAL TOPOLOGY

Main Site:

- Router: Cisco 2911 (UrbanMart_Main_Router)

- Distribution Switch: Cisco 3650 (UrbanMart_Main_Dist)

- Access Switch: Cisco 2960 (UrbanMart_Main_Access)

- End Devices: pc_0 to pc_4, printer_0 to printer_3

- Connected to the Cloud for WAN access

Branch Site:

- Router: Cisco 2911 (UrbanMart_Branch_Router)

- Distribution Switch: Cisco 3650 (UrbanMart_Branch_Dist)

- Access Switch: Cisco 2960 (UrbanMart_Branch_Access)

- End Devices: pc_5 to pc_8, printer_4 to printer_7

Cloud:

- Simulates WAN connectivity between the two routers

# VLAN CONFIGURATION

- VLAN 10 – Management (Switch & Router Management)

- VLAN 20 – Sales Department (pc_0–pc_2, printer_0–printer_1)

- VLAN 30 – Inventory Department (pc_3–pc_4, printer_2–printer_3)

- VLAN 40 – IT Department (pc_5–pc_6, printer_4–printer_5)

- VLAN 50 – Guest Wi-Fi (pc_7–pc_8, printer_6–printer_7)

# IP ADDRESSING

- VLAN 10: 192.168.10.0/24, Gateway 192.168.10.1

- VLAN 20: 192.168.20.0/24, DHCP pool .100–.120

- VLAN 30: 192.168.30.0/24, DHCP pool .100–.120

- VLAN 40: 192.168.40.0/24, DHCP pool .100–.120

- VLAN 50: 192.168.50.0/24, Static Range .200–.220

- WAN Link: 10.0.0.0/30 between main and branch routers

- Static IPs used for all servers and router interfaces

# 6. PROJECT TIMELINE

| Phase | Duration | Deliverables |
| --- | --- | --- |
| Requirements Gathering | 2 days | Final list of devices, IP plan, VLAN schema. |
| Network Design | 2 days | Logical and Physical Topology Diagrams |
| Device Configuration | 3 days | Cisco configurations and .pkt simulation file |
| Testing & Validation | 2 days | Ping tests, routing validation, screenshots |
| Final Documentation | 1 day | Complete project report and network manual |
| Total Estimated Time: | 10 working days |  |

# 7. RISKS AND MITIGATION

| Risk | Mitigation Strategy |
| --- | --- |
| VLAN misconfiguration | Validate VLAN map and use simulation testbed |
| Overlapping IP addressing | Use planning spreadsheet and peer review |
| Routing misconfigurations | Implement and test OSPF or RIP with redundancy |
| Simulation software crash or bug | Frequent save and version control |

# 8. CONCLUSION

This updated proposal reflects the final network simulation built in Packet Tracer. It includes complete logical topology with two interconnected sites (main and branch) using Cisco routers and switches, nine PCs, eight printers, and four servers. Cloud connectivity enables WAN simulation and routing, while VLANs and ACLs ensure secure segmentation. The design ensures reliable, testable, and expandable architecture suitable for eventual physical deployment.